City Of Detroit

Special Provision

for

# RECESSING for PAVEMENT MARKINGS

DPW / TED: JF 																1 of 1								  										12-09-13

A. Description. Recessing grooves shall be cut into the roadway to facilitate the protection of pavement marking material from vehicle wear and cleaning equipment such as snow plows. This work shall be performed for the installation of all transverse line markings except where markings are in-laid into fresh hot asphalt. This work shall conform to Section 811 of the Standard Specifications for Construction and the following provisions specified herein.

B. Construction.

The groove recesses shall be made on concrete and asphalt road surfaces in a single dry pass cut and the equipment used shall be self-vacuuming and leave the cut groove ready for material installation. The blade tool shall be made of stacked diamond cutting heads or engineer approved flat-tooth design hard alloy material such as tungsten-carbide. The bottom of the groove shall have a fine corduroy finish. The remaining roadway surface shall be solid and intact and not cracked or broken with potentially loose pieces subject to breaking off. If a coarse tooth pattern or cracked road surface results, then the tooling must be changed. Either a) the number of blades is to be increased and the spaces on the cutting head are to be decreased or b) the type of blade is to be changed to a superior material or cutting tool design.

Recess Width: 						Material width +1 inch, + / - 1/8 inch

Recess Depth: 						As recommended by the manufacturer, + / - 5 mils

Permanent Marking Material Application:

The recessing equipment and method used shall comply with the pavement material manufacturer’s installation instructions. Recess grooves shall be clean and dry prior to placing any pavement marking material. The entire pavement marking must be placed within the recess and shall be placed in the same day as the recessing work.

C. Measurement and Payment. The completed work as described will be paid for at the contract unit price for the following contract pay items:

## Contract Item (Pay Item) 																						Pay Unit

_ Recessing for Pavt Mrkg, 06 inch	…………………………………….…	Foot

_ Recessing for Pavt Mrkg, 12 inch	………………………………….……	Foot

_ Recessing for Pavt Mrkg, 18 inch	…………………….……..………….	Foot

_ Recessing for Pavt Mrkg, 24 inch	………………………………….……	Foot

The item of Recessing Pavement Markings includes all equipment and labor necessary to complete the work. Payment for any temporary paint lines (used as a template) is included in the item Recessing Pavement Markings.