CITY OF DETROIT

SPECIAL PROVISIONS

FOR

COLD MILLING HMA SURFACE, MODIFIED

1 of 1

DET: UP									        Revised: 05-08-12

Description	This work shall consist of removing HMA pavement or surface by use of cold-milling equipment to prepare the foundation for HMA overlays per section 501.03 of the 2012 Michigan Department of Transportation (MDOT) Standard Specifications for Construction and as specified herein.

The cost of exploration for the existing street car rails when encountered during cold milling shall be included in the work of cold milling HMA surface. Extra payment for cost of actual removal and disposal of any street car rail will be made per section 103.02

The Contractor shall schedule the cold-milling operation so that entire lane width of the roadway is cold milled between road terminus limit   before switching over to other lane/side. This is required to avoid having a vehicle travel over milled surface on one side and normal pavement surface other side.

At the end of the day, the Contractor shall place a wedge, a minimum of 6 feet wide, the full length of all transverse and longitudinal joints at intersections where milling is greater than 2 inches in depth, to provide a smooth transition onto and off of the milled surface.

It is the Contractor’s responsibility to establish the finish (final) grade on all streets and to ensure proper (positive) drainage at all locations.  This work shall included in the work of cold milling and be done at no additional cost to project.

Measurement and Payment The completed work as measured for “Cold Milling HMA Surface, Modified” will be paid for at the contract unit price for the following contract pay item.

Cold milling of existing concrete patches, and establishing finish (final) grade to ensure proper drainage, the cost of exploration for an existing street car rail when encountered during cold milling and wedging for longitudinal and transverse transition when required is included in the work of "Cold Milling HMA Surface, Modified".

The price shall be payment in full for removing the HMA material to the depth specified on the typical cross section sheet, and for transportation and disposal of the material.

Pay Item									Pay Unit

Cold Milling HMA Surface, Modified						Square Yard