CITY OF DETROIT

SPECIFICATION

FOR

COLD MILLING CONCRETE SURFACE

DET: GJM 										   01-13-2016

Description:  This work shall consist of removing the top portions of an existing concrete pavement surface to the depth and cross section as shown on the detail drawing in these Specifications in order to improve longitudinal/transverse grades, surface texture, or to prepare a foundation for the placing of HMA surfacing courses.

Equipment:  The equipment for this work shall be capable of accurately establishing profile grades by referencing from either the existing pavement or from an independent grade control.  The equipment shall have an effective means of providing an acceptable surface texture, of removing chips from the pavement, and for preventing dust from escaping into the air.

Construction:  The concrete surface shall be removed to the depth, width, grade and cross-section as shown on the Cold Milling Concrete Surfaces and Paving detail in these Specifications, and as directed by the Engineer.  Provide a surface reasonably smooth and free of gouges, holes or other defects caused by milling.  Fill and compact gouges, holes and defects by hand patching in accordance with Section 501.03.C.9 of the 2012 MDOT Standard Specifications for Construction.  Repair all damage to abutting surfaces as directed by the Engineer.  All costs to correct milling defects and damages shall be borne by the Contractor.

The material removed from the pavement surface shall be collected and disposed of as specified in Sections (104.07.D) and 204.03.B of the 2012 MDOT Standard Specifications for Construction.

Measurement and Payment:  “Cold Milling Conc Surface” will be measured by area in square yards and paid for at the contract unit price.  This pay item shall include all labor, equipment and materials required to remove the concrete surface and dispose of the excess materials.

Pay Item								Pay Unit

Cold Milling Conc Surface						Square Yard