CITY OF DETROIT

SPECIFICATIONS

FOR

DRAINAGE STRUCTURE CLEANING AND SEWER LEAD CLEANING

DET: GJM           								                 10-02-15

Description:  The work of drainage structure cleaning consists of cleaning existing catch basins and manholes in the area of the project where directed by the Engineer. The work of sewer cleanout consists of cleaning sewer lines between catch basins and between catch basins and manholes.

Construction:  When necessary and as directed by the Engineer, the work shall be performed in accordance with Section 403.3.G of the 2012 MDOT Standard Specifications for Construction and as modified herein.

Cleaning shall be by high velocity hydro-cleaning (jetting).  Mechanical cleaning shall only be used when authorized by the Engineer.  Contractor shall take precautions to protect the sewer lines from damage.

No debris shall be accumulated on site except in totally enclosed containers approved by the Engineer. All material deposits shall be removed from site and disposed of at an approved location.

Some of the catch basins, manholes and sewers may be completely filled. The Engineer shall determine actual condition and necessity for cleaning prior to cleanout. Cleaning shall restore 90% of the pipe's carrying capacity.

Measurement and Payment:  "Dr Structure Cleaning, Modified" will consist of cleaning catch basins and manholes, and shall be paid for at the contract unit price each, and will include all equipment and labor necessary to clean each catch basin or manhole, including installation of bulkheads as necessary and hauling away and disposing of any waste and debris generated.

"Dr Structure Lead, Cleaning, Modified" consists of cleaning sewers between catch basins or between catch basins and manholes, and shall be paid for at the contract unit price per foot which will include payment for furnishing all equipment and labor to clean sewers lines regardless of size, temporary bulkheads, and hauling away and disposing of any waste and debris generated.

Pay Item									Pay Unit

Dr Structure Cleaning, Modified				  	             Each

Dr Structure Lead, Cleaning, Modified					 Foot