CITY OF DETROIT

SPECIFICATION

FOR

EROSION AND SEDIMENTATION CONTROL

DET: GJM                                                                                                         12-17-15

Description:  This specification covers the protection of drainage structures, and storm and sanitary systems, from the effects of construction activities, erosion and sedimentation entering into drainage structures during the course of the work.

Material:  Materials shall be as specified in Section 208.02 Materials of the 2012 MDOT Standard Specifications for Construction.

Construction:  Erosion and sedimentation control shall be in accordance with Section 208.03.C.08 of the 2012 MDOT Standard Specifications for Construction, and as follows.

Prior to starting work on a particular street or block, all drainage structure covers on that street or block, or those that the street being worked on drains to, shall be wrapped with a geotextile filter fabric.  The fabric shall allow water to flow through, but shall prevent the passage of silts, dirt, clay stone and other construction debris.  The fabric shall be wrapped over the top of the cover and extended under, such that when the cover is placed back in the frame, the fabric is around the entire perimeter between the frame and cover.

During final clean-up of the area, the fabric shall be carefully removed, preventing any captured debris from falling into the drainage structure.  The fabric shall be inspected daily and repaired immediately to insure the complete and consistent effectiveness of the erosion and sedimentation control.

Measurement and Payment:  Erosion Control, Inlet Protection, Geotextile and Stone shall be paid for at the Contract Unit Rate in accordance with Section 208.04.G.2 of the 2012 MDOT Standard Specifications for Construction.

Erosion Control, Maintenance, Sediment Removal shall be paid for at the Contract Unit Rate in accordance with Section 208.04.L of the 2012 MDOT Standard Specifications for Construction.

Pay Item									Pay Unit

Erosion Control, Inlet Protection, Geotextile and Stone			Each

Erosion Control, Maintenance, Sediment Removal				Cubic Yard