CITY OF DETROIT

SPECIFICATIONS

FOR

SIDEWALK RAMP, ADA, AND

DETECTABLE WARNING SURFACE

DET-GJM						 		                        09-28-2015

Description: This special provision covers construction of sidewalk ramp with detectable warning surface.

The work consist of constructing sidewalk ramp and/or retrofitting ramp with detectable warning surface in accordance with the applicable provisions of Section 803 of the 2012 MDOT Standard Specifications for Construction and the current MDOT Standard Plan R-28-I, as modified.

Complete this work in conformance with the American with Disability Act (ADA) provisions as applicable.

Material:  The Detectable Warning Surface color shall be will be brick red, Federal Color Number 22144, unless otherwise approved by the Engineer. The color shall be homogeneous throughout the tile

Construction: Construct sidewalk ramp(s), landings flares and detectable warning surface according to MDOT Standard Plan R-28-I.  The concrete sidewalk and ramp thickness shall be as determined by the City of Detroit and as shown on the MDOT Sidewalk Ramp and Detectable Warning Details, R-28-H, included in these Specifications. The Detectable Warning Surface installation shall also comply with the manufacturer’s instructions.

The curb and gutter section at the bottom of the ramp (ramp opening) shall be a full depth curb with a minimum depth of 12 inches, and shall be 18 inches from the back of curb to abutting pavement.

It is also the Contractor’s responsibility to incorporate any changes made to the ADA accessibility requirements that may take effect prior to the start date of actual construction. If the Contractor determines that any changes significantly alter the original bid cost, the Contractor may submit a written request to the Engineer or his representative for approval and compensation. Include a cost comparison between the original bid cost and the cost of the ADA required changes in the request.

The contractor shall stamp all concrete with a legible stamp bearing the name of the company and the year of construction. No sidewalk or ramp construction shall commence without a Contractor’s stamp on site.

Restore berm areas disturbed beyond sidewalk ramp repair limits to its original conditions. No separate payment will be made for such restoration, but is considered part of the work item.

Any constructed ramps that do not met the current MDOT Sidewalk Ramp Standard Plans and the latest ADA accessibility requirements will be removed and replaced by the Contractor, as directed by the Engineer or his representative, at the Contractor’s expense.

Measurement and Payment: The completed work as described herein for Sidewalk Ramp, ADA and Detectable Warning Surface, will be measured by as square feet and feet respectively.  The contract unit price will be payment in full for furnishing all materials, labor and equipment necessary to construct the ADA ramp and curb and warning surface.

Pay Item 								 	Pay Unit

Sidewalk Ramp, ADA, __ inch						Square Feet

Detectable Warning Surface							Feet