CITY OF DETROIT

SPECIFICATION

FOR

SIDEWALK WITH TREE ROOTS

DET: GJM                                                                                                         12-17-15

Description:  This special provision covers the removal and replacement of sidewalk and driveways which has been raised and/or damaged by tree roots and includes the trimming of the tree roots in conjunction with the sidewalk and driveway replacement.

Construction:  This item of work includes removing the concrete pavement, cutting and removing tree roots, removing wood chips and all debris material and properly disposing of same off site; backfilling, compacting, and grading of the disturbed area; and construction of concrete sidewalk and driveway.

The concrete sidewalk and driveway shall be removed in accordance with the MDOT Standard Specifications for Construction, Section 204, Removing Miscellaneous Structures and Materials.

All roots or stumps under the sidewalk and driveway area and within four (4) inches of the edges of the sidewalk or driveway, shall be ground to a depth that severs the roots from the main root mass or to a minimum depth of eight (8) inches from the proposed finished grade of the top of the sidewalk.  All voids resulting from the removal of roots or stumps shall be backfilled within one (1) day of removal.  The Contractor shall fill the voids with clean, compacted backfill as specified in MDOT Standard Specifications for Construction, Section 204.03.C.  All debris resulting from the grinding process shall be removed from the site and cannot be used as fill.  All roots and grindings shall be removed and disposed of within three (3) days of removal/grinding.

Concrete sidewalk and driveway of the thickness specified shall be constructed in accordance with Section 803 of the 2012 Michigan Department of Transportation Standard Specifications for Construction.  Sidewalk cross slopes shall be between 1% and 2%, sloping towards the street.

Measurement and Payment:  "Sidewalk, Conc, __ inch, With Tree Roots, Modified" of the thickness specified shall be measured in square feet.  Payment shall be for the entire flag or flags of sidewalk, or driveway affected by tree roots as marked by the Engineer. The contract unit price shall be payment in full for furnishing all labor, equipment and material necessary to complete the installation of new concrete sidewalk and driveway.

Pay Item									Pay Unit

Sidewalk, Rem								Square Yard

Sidewalk, Conc, __ inch, With Tree Roots, Modified				Square Feet