CITY OF DETROIT

SPECIFICATION

FOR

SIDEWALKS

DET: GJM                                                                                                         12-17-15

Description:  This special provision covers the removal and replacement of sidewalk and driveways.

Construction:  This item of work includes removing the concrete pavement, backfilling, compacting, and grading of the disturbed area, and construction of concrete sidewalk and driveway.

The concrete sidewalk and driveway shall be removed in accordance with the MDOT Standard Specifications for Construction, Section 204, Removing Miscellaneous Structures and Materials.

Concrete sidewalk and driveway, of the thickness specified, shall be constructed in accordance with Section 803 of the 2012 Michigan Department of Transportation Standard Specifications for Construction.  Sidewalk cross slopes shall be between 1% and 2%, sloping towards the street.

Measurement and Payment:  Concrete sidewalks and driveways of the thickness specified shall be measured in square feet.  Payment shall be for the entire flag or flags of sidewalk, or driveway, as marked by the Engineer. The contract unit price shall be payment in full for furnishing all labor, equipment and material necessary to complete the installation of new concrete sidewalk and driveway.

Pay Item									Pay Unit

Sidewalk, Rem								Square Yard

Sidewalk, Conc, __ inch							Square Feet