- Description. This work consists of removing sign posts as shown on the plans and as directed by the Engineer., whether separate or integral with the pavement or sidewalk, where the adjoining pavement or sidewalk remains.

- Materials. Provide necessary materials per 2012 MDOT Standard Specifications for Construction.

- Construction. The work shall be done in accordance with the applicable provisions of the 2012 MDOT Standard Specifications for Construction.

Post removal shown on the plans that is located on existing concrete or asphalt surface in good condition that is to remain shall include cutting the sign post flush at existing grade or cutting bolts flush at existing grade and incidental filling of voids with grout and will not be paid for separately.

Post removal shown on the plans that is located on existing concrete or asphalt surface in poor condition that is to remain shall include cutting and removing adjacent pavement, minimum 12” each way, and replacing pavement in kind to match existing and will not be paid for separately.

Post removal shown on the plans that is located within lawn or green space shall include site restoration in kind to match existing for area disturbed for post removal and will not be paid for separately.

- Measurement and Payment.  The completed work, as described, including sawcutting, excavation, backfilling, pavement replacement and/or site restoration will be measured and paid for at the contract unit price for the following pay item:

Pay Item 	Pay Unit

_Post, Rem	Each